Prototype RWIS Alert System Test Plan

## Montana Tech Senior Design Team

Date:

February 13, 2026

Authors:

Quinn Cox

Montana Tech

email: qcox@mtech.edu

Hayden Pride

Montana Tech

email: hpride@mtech.edu

Nick Steilman

Montana Tech

email: nsteilman1@mtech.edu

Douglas Galarus

Assistant Professor, Montana Tech

email: dgalarus@mtech.edu

## 1	REFERENCES

Prototype RWIS Alert System Design Cox, Q., Pride, H., Steilman, N., Galarus, D. November 13, 2024.

Prototype RWIS Alert System Requirements Specifications (SRS), Cox, Q., Pride, H., 	Steilman, N., Galarus, D. October 28, 2024.

Test Plan Outline (IEEE 839 Format) by System Evolutif Limited.

IEEE Standard for Software and System Test Documentation by IEEE Computer Society July 18, 2008.

## 2 	INTRODUCTION

This is the test plan for the Prototype RWIS Alert System, which is a standalone application focused on processing Caltrans RWIS data to generate condition-based alerts. The system retrieves and parses real-time RWIS data, stores it locally, and evaluates user-defined alert conditions against the incoming data. Users can create and manage alert definitions, view live RWIS data, and observe which alerts are active based on current environmental conditions.

## 3	TEST ITEMS

This section identifies the specific software components, data structures, and 	subsystems slated for testing. These items are derived from the Prototype RWIS 	Alert System Design and Software Requirements Specification (SRS).

### 3.1	Backend Data Processing (Python)

These components handle the core logic: retrieving data, normalizing it, and evaluating alerts. If these fail, the frontend display is irrelevant.

#### 3.1.1	dataRetrievalModule:

Description: The Python script responsible for connecting to the Caltrans CWWP2 portal and downloading raw RWIS CSV files.

Test Focus: Verifying successful connection, handling network timeouts (simulated), and ensuring the script runs on the correct 10-minute schedule.

#### 3.1.2	dataParsingEngine:

Description: The logic that reads raw CSV data into memory, validates headers/row counts, and performs unit conversions (e.g., Celsius to Fahrenheit).

Test Focus: Accuracy of unit conversions, handling of "null" or missing sensor values, and rejection of malformed data rows.

#### 3.1.3	alertEvaluationLogic:

Description: The engine that compares normalized sensor data against user-defined thresholds to determine if an alert should be Actuated or De-actuated.

Test Focus: Boolean logic checks (AND/OR), boundary value analysis (e.g., data exactly matching the threshold), and alert duration logic.

### 3.2 	Database & Storage Components

### Testing here ensures data is stored securely and persists correctly across 		sessions.

#### 3.2.1	sqliteDatabaseSchema:

Description: The relational database file containing the users, alert_defs, and 	alert_history tables.

Test Focus: Referential integrity (e.g., deleting a user also removes their alerts) and 	correct storage of encrypted passwords.

#### 3.2.2	flatFileStorage:

Description: The local latest_rwis.csv file generated after processing, which serves 	as the source for the web display.

Test Focus: Verifying the file is overwritten (not appended) during updates and that 	file permissions allow the PHP interface to read it.

### 3.3 	Web Interface (PHP)

These items are the user-facing elements. Testing focuses on usability and 		security.

#### 3.3.1	userAuthenticationModule:

Description: PHP scripts managing login, session creation, logout, and password 	hashing.

Test Focus: Session persistence, preventing access to internal pages without login, 	and correct hashing of credentials.

#### 3.3.2	alertConfigurationInterface:

Description: The web forms allow users to create, edit, and delete alert 		definitions.

Test Focus: Input validation (e.g., preventing invalid characters in names) and 	ensuring dropdowns populate with the correct sensor list.

#### 3.3.3	rwisDataDashboard:

Description: The tabular display of current RWIS data.

Test Focus: Visual clarity when data is missing/unavailable and ensuring the 		displayed values match the flatFileStorage.

## 4 	SOFTWARE RISK ISSUES

This section outlines potential software risks that could impact the testing process or the stability of the prototype.

A.	Caltrans Data Format Changes: The system relies on a specific CSV column structure from the CWWP2 portal.

B.	Database Locking (SQLite): Since SQLite is a file-based database, there is a risk of "database is locked" errors if the Python backend tries to write data at the exact same moment the PHP frontend tries to read it.

C.	Network Latency/Timeouts: The dataRetrievalModule depends on an external connection. High latency could cause the script to hang indefinitely.

## 5	FEATURES TO BE TESTED

The following is a list of the functions and system capabilities that will be focused on during testing.

A. Automated Data Retrieval: Verification that the dataRetrievalModule successfully connects to the CWWP2 portal and downloads the latest CSV data at the defined 10-minute intervals.

B. Data Parsing & Normalization: Verification that raw CSV data is correctly read into memory, valid rows are processed, and units are converted.

C. Alert Evaluation: Verification that the system correctly identifies when incoming data activates a user-defined threshold and when it returns parameters outside of the alert.

D. User Authentication: Verification of secure login, session persistence, and logout functionality.

E. Alert Configuration: Verification that users can Create, Read, Update, and Delete alert definitions via the web interface.

F. Data Dashboard: Verification that the web interface accurately displays the most recent RWIS data as stored in the flat file/database.

## FEATURES NOT TO BE TESTED

The following features are explicitly out of scope for this test plan.

- External Notification Delivery: We will not test the actual delivery of SMS or Emails via third-party gateways. Testing stops at the generation of the internal alert log.

- Mobile Application: A dedicated mobile app is out of scope.

- Caltrans Infrastructure: We assume the Caltrans CWWP2 server is functional. We will simulate downtime locally, but we are not load-testing their servers.

- Historical Data Analysis: The system is designed to show current conditions; long-	term historical weather analysis is not being tested.

## 7	APPROACH (STRATEGY)

### 7.1     Testing Levels

Testing for the Prototype RWIS Alert System will consist of system and integration testing to ensure the system operates as intended using the data from the CWWP2 online portal. Due to time constraints, majority of testing will be conducted between February 11, 2026, and February 20, 2026, by the Senior Design Team.

System Testing: This will be conducted by the Senior Design Team. The team will validate the end-to-end flow of data by ingesting CSV files retrieved from the CWWP2 online portal. The focus is ensuring the system can correctly parse, store, and display valid Caltrans data formats from various stations without error.

### 7.2     Test Tools

- CWWP2 Online Portal: The primary source for retrieving raw RWIS CSV 	files for testing.

- SQLite Database Browser: A GUI tool used to inspect the backend 	alert_history and users tables to verify data persistence.

- Python Retrieval Scripts: The project's own dataRetrievalModule will be used 	to fetch and save these files for analysis.

### 7.3     System Tests

The following are descriptions of the primary test categories used to validate the system functionality.

- Data Ingestion Verification: A functional test to confirm that the system correctly parses headers and row data from the files downloaded via the CWWP2 online portal.

- Alert Logic Validation: A logic test where specific values are chosen to trigger boundaries. For example, we will use a temperature of 32.0°F to ensure the logic handles "Less Than or Equal To" operators correctly compared to a value of 32.1°F.

### File Selection

### Files will be sourced directly from the CWWP2 online portal.

### Selection Criteria: Files will be selected from multiple Caltrans districts to 	ensure the system processes data correctly across all locations, regardless 	of minor regional differences in file formatting or sensor availability.

### Scope: Due to the large volume of available data, the team will select a 	representative sample of files (approx. 3-5 files per district) for direct 	testing.

### Analysis: These selected files will be fed into the system, and the output 	(Web 	Dashboard and Database) will be analyzed to ensure all fields were 	mapped and stored correctly.

### 7.5     Measures and Metrics

The following will be collected during system testing.

### Defects: The number of parsing errors or "NULL" values encountered when 	processing the portal files.

### District Coverage: The number of distinct districts successfully tested.

### Pass/Fail Rate: The percentage of selected files that processed without 	triggering an application error.

## 8	ITEM PASS/FAIL CRITERIA

A test case is considered a "Pass" if the system successfully ingests the WeatherShare data, stores it in the database, and displays the correct values on the dashboard without error. For alert logic, a "Pass" means the system correctly identifies whether the real-world data violates the user-defined threshold.

A test case is considered a "Fail" if:

- The parser crashes on a valid CSV file from the portal.

- The dashboard displays "NULL" or empty values for valid sensor readings.

- An alert fails to trigger when the data clearly violates the threshold.

## 9	SUSPENSION CRITERIA AND RESUMPTION REQUIREMENTS

There were no criteria identified that required a suspension of the project.

## 10 	TEST DELIVERABLES

Testing Plan

Testing Results

## 11	REMAINING TEST TASKS

| TASK | Assigned To | Status |
| --- | --- | --- |
| Create and Approve System Tests | Project manager, Senior design team | In Progress (02/10/2025) Complete (02/21/2025) |
| Execute System Tests | Senior design team | Complete (02/21/2025) |
| System Integration Testing | Project Manager, Senior Design Team | Team portion: Complete (02/21/2025) |

## 12	ENVIRONMENTAL NEEDS

The following elements are required to support the overall testing efforts.

  - Data Source: Access to the CWWP2 online portal to retrieve real-world RWIS CSV files for testing

  - Software Stack: Apache Web Server, PHP, Python, and SQLite

  - Hardware: A standard PC or Laptop (Windows/Linux/macOS) capable of running a local web server.

## 13	RESPONSIBILITIES

|  | Design Team |
| --- | --- |
| User Account Verification | X |
| Output Verification | X |
| Testing Documentation | X |
| Testing Execution | X |

## 14	SCHEDULE

Initial testing will be completed by the Senior Design Team between February 11, 2026, and February 20, 2026. Any testing beyond this date will need to be conducted by a different team.

## 15	PLANNING RISKS AND CONTINGENCIES

There were no planning risks and contingencies identified.

## 16	APPROVALS

| Project Manager – Douglas Galarus |  |
| --- | --- |
| Senior Design Team - Unanimous | Approved by Quinn Cox, Hayden Pride, Nick Steilman |